## China's Health and Environment: Air pollution and health effects

## http://www.wri.org/publication/content/8416

### Poor ambient air quality prevails

“The residents of many of China’s largest cities are living under long-term, harmful air quality conditions,” Zhao Weijun, deputy director of the air pollution department of NEPA, reported in 1997 in China Environment News [16]. China has long recognized air pollution as a critical problem. Ambient concentrations of total suspended particulates (TSP) and sulfur dioxide (SO2) are among the world’s highest. (See China’s Air Pollution Levels Are Among the World’s Highest.) In 1995, more than one half of the 88 cities monitored for SO2 were above the World Health Organization (WHO) guideline. All but two of the 87 cities monitored for TSP far exceeded WHO’s guideline. Some cities such as Taiyuan and Lanzhou had SO2 levels almost 10 times the WHO guideline [17].

Largely because of controls at power plants and within households, particulate emissions have not risen as much as might have been expected with the doubling of coal consumption. Overall, particulate emissions in China have remained relatively level since the early 1980s [18]. In fact, in some large cities, ambient particulate concentrations have decreased markedly since the 1980s [19]. In contrast, SO2 emissions have roughly paralleled the increase in coal consumption, reflecting heavy coal burning and inadequate sulfur control measures.

Coal burning, the primary source of China’s high SO2 emissions, accounts for more than three quarters of the country’s commercial energy needs, compared with 17 percent in Japan and a world average of 27 percent [20]. China’s consumption of raw coal increased annually by 2 percent between 1989 and 1993 [21]. Meanwhile, SO2 emissions increased by more than 20 percent and TSP increased by approximately 10 percent [22]. The country is expected to burn 1.5 billion metric tons of coal annually by the year 2000, up from 0.99 billion metric tons in 1990 [23]. Without even more dramatic measures to control emissions than are currently in place, the deterioration of air quality seems inevitable.

China’s Air Pollution Levels Are Among the World’s Highest

Ambient concentrations of air pollutants, 1995

Source: The World Bank, Clear Water, Blue Skies: China’s Environment in the New Century (The World Bank, Washington, D.C., 1997), Figure 1.1, p. 6.